# Blocko Block Agent – Command Reference

This document describes the UART command protocol for the Blocko Block Agent (Agent 2). The Block Agent programs blank AT24C256 EEPROMs into Blocko code blocks via a PCA9548A I²C multiplexer with 8 channels (0–7).

# Communication

Commands are sent as newline-delimited JSON over UART at 115200 baud (8N1). Each command must be a single line ending with \n. Responses are returned as single-line JSON.

Connecting to the Block Agent:

screen /dev/ttyACM0 115200

picocom -b 115200 /dev/ttyACM0

idf.py -p /dev/ttyACM0 monitor

# EEPROM Data Structure (32 bytes)

Each programmed EEPROM contains the following layout:

| Address | Size | Field | Description |
| --- | --- | --- | --- |
| 0x0000 | 1 | Type ID | Block type identifier (see Block Types table) |
| 0x0001 | 1 | Subtype | Block subtype for variants |
| 0x0002 | 1 | Param 1 | Block-specific parameter 1 |
| 0x0003 | 1 | Param 2 | Block-specific parameter 2 |
| 0x0004–0x0007 | 4 | Serial Number | Auto-generated unique ID (MAC[4:5] + counter) |
| 0x0008 | 1 | Version | Always 0x01 |
| 0x0009 | 1 | Checksum | XOR of bytes 0x00–0x08 |
| 0x000A–0x000F | 6 | Reserved | Set to 0x00 |
| 0x0010–0x001F | 16 | Name | ASCII, null-terminated (max 15 chars) |

# Common Fields

All commands share these fields:

| Field | Type | Required | Description |
| --- | --- | --- | --- |
| cmd | string | Yes | Command name (WRITE_BLOCK, READ_BLOCK, ERASE_BLOCK, VERIFY_BLOCK, BATCH_PROGRAM) |
| channel | int (0–7) | No | PCA9548A mux channel. Selects which EEPROM slot to target. Defaults to 0 if omitted. |

# Commands

## WRITE_BLOCK

Writes block configuration data to the EEPROM at the specified channel. Auto-generates a unique serial number and XOR checksum. Verifies data by reading back after write.

| Field | Type | Required | Description |
| --- | --- | --- | --- |
| cmd | string | Yes | "WRITE_BLOCK" |
| channel | int | No | Mux channel 0–7 (default 0) |
| type | int | Yes | Block Type ID (see Block Types table). Decimal or hex value. |
| subtype | int | No | Block subtype for variants (default 0) |
| param1 | int | No | Extra parameter byte 1, block-specific (default 0) |
| param2 | int | No | Extra parameter byte 2, block-specific (default 0) |
| name | string | No | Human-readable label, max 15 characters (default "") |

### WRITE_BLOCK Examples

Event blocks:

Program a START block on channel 0

{"cmd":"WRITE_BLOCK","channel":0,"type":1,"name":"Start"}

Response:

{"response":"WRITE_OK","type":1,"serial":"A1B20000"}

Program a WHEN_BUTTON block on channel 1

{"cmd":"WRITE_BLOCK","channel":1,"type":2,"name":"When Button"}

Response:

{"response":"WRITE_OK","type":2,"serial":"A1B20001"}

Program a WHEN_SENSOR block on channel 2

{"cmd":"WRITE_BLOCK","channel":2,"type":3,"name":"When Sensor"}

Response:

{"response":"WRITE_OK","type":3,"serial":"A1B20002"}

Control blocks – Wait:

Program a WAIT_1S block (default channel 0)

{"cmd":"WRITE_BLOCK","type":16,"name":"Wait 1s"}

Response:

{"response":"WRITE_OK","type":16,"serial":"A1B20003"}

Program a WAIT_2S block on channel 4

{"cmd":"WRITE_BLOCK","channel":4,"type":17,"name":"Wait 2s"}

Response:

{"response":"WRITE_OK","type":17,"serial":"A1B20004"}

Program a WAIT_5S block on channel 5

{"cmd":"WRITE_BLOCK","channel":5,"type":18,"name":"Wait 5s"}

Response:

{"response":"WRITE_OK","type":18,"serial":"A1B20005"}

Control blocks – Repeat:

Program a REPEAT_2X block

{"cmd":"WRITE_BLOCK","type":32,"name":"Repeat 2x"}

Response:

{"response":"WRITE_OK","type":32,"serial":"A1B20006"}

Program a REPEAT_3X block

{"cmd":"WRITE_BLOCK","type":33,"name":"Repeat 3x"}

Response:

{"response":"WRITE_OK","type":33,"serial":"A1B20007"}

Program a REPEAT_5X block

{"cmd":"WRITE_BLOCK","type":34,"name":"Repeat 5x"}

Response:

{"response":"WRITE_OK","type":34,"serial":"A1B20008"}

Control blocks – Flow:

Program an IF block

{"cmd":"WRITE_BLOCK","type":48,"name":"If"}

Response:

{"response":"WRITE_OK","type":48,"serial":"A1B20009"}

Program an ELSE block

{"cmd":"WRITE_BLOCK","type":49,"name":"Else"}

Response:

{"response":"WRITE_OK","type":49,"serial":"A1B2000A"}

Program an ENDIF block

{"cmd":"WRITE_BLOCK","type":50,"name":"End If"}

Response:

{"response":"WRITE_OK","type":50,"serial":"A1B2000B"}

Program a LOOP_FOREVER block

{"cmd":"WRITE_BLOCK","type":64,"name":"Loop Forever"}

Response:

{"response":"WRITE_OK","type":64,"serial":"A1B2000C"}

Sensing blocks:

Program an IF_BUTTON block on channel 3

{"cmd":"WRITE_BLOCK","channel":3,"type":80,"name":"If Button"}

Response:

{"response":"WRITE_OK","type":80,"serial":"A1B2000D"}

Program an IF_LIGHT_HIGH block

{"cmd":"WRITE_BLOCK","type":81,"name":"If Light Hi"}

Response:

{"response":"WRITE_OK","type":81,"serial":"A1B2000E"}

Program an IF_LIGHT_LOW block

{"cmd":"WRITE_BLOCK","type":82,"name":"If Light Lo"}

Response:

{"response":"WRITE_OK","type":82,"serial":"A1B2000F"}

Program an IF_SOUND block

{"cmd":"WRITE_BLOCK","type":83,"name":"If Sound"}

Response:

{"response":"WRITE_OK","type":83,"serial":"A1B20010"}

Program an IF_DISTANCE block

{"cmd":"WRITE_BLOCK","type":84,"name":"If Distance"}

Response:

{"response":"WRITE_OK","type":84,"serial":"A1B20011"}

Program an IF_TEMP_HIGH block

{"cmd":"WRITE_BLOCK","type":85,"name":"If Temp Hi"}

Response:

{"response":"WRITE_OK","type":85,"serial":"A1B20012"}

LED blocks:

Program an LED_ON block on channel 6

{"cmd":"WRITE_BLOCK","channel":6,"type":112,"name":"LED On"}

Response:

{"response":"WRITE_OK","type":112,"serial":"A1B20013"}

Program an LED_OFF block

{"cmd":"WRITE_BLOCK","type":113,"name":"LED Off"}

Response:

{"response":"WRITE_OK","type":113,"serial":"A1B20014"}

Program an LED_RED block on channel 5

{"cmd":"WRITE_BLOCK","channel":5,"type":114,"name":"LED Red"}

Response:

{"response":"WRITE_OK","type":114,"serial":"A1B20015"}

Program an LED_GREEN block

{"cmd":"WRITE_BLOCK","type":115,"name":"LED Green"}

Response:

{"response":"WRITE_OK","type":115,"serial":"A1B20016"}

Program an LED_BLUE block on channel 7

{"cmd":"WRITE_BLOCK","channel":7,"type":116,"name":"LED Blue"}

Response:

{"response":"WRITE_OK","type":116,"serial":"A1B20017"}

Sound blocks:

Program a BEEP block

{"cmd":"WRITE_BLOCK","type":128,"name":"Beep"}

Response:

{"response":"WRITE_OK","type":128,"serial":"A1B20018"}

Program a BEEP_LONG block

{"cmd":"WRITE_BLOCK","type":129,"name":"Beep Long"}

Response:

{"response":"WRITE_OK","type":129,"serial":"A1B20019"}

Program a PLAY_HAPPY block

{"cmd":"WRITE_BLOCK","type":130,"name":"Play Happy"}

Response:

{"response":"WRITE_OK","type":130,"serial":"A1B2001A"}

Program a PLAY_SAD block

{"cmd":"WRITE_BLOCK","type":131,"name":"Play Sad"}

Response:

{"response":"WRITE_OK","type":131,"serial":"A1B2001B"}

Motion blocks:

Program a MOTOR_FWD block on channel 2

{"cmd":"WRITE_BLOCK","channel":2,"type":144,"name":"Motor Fwd"}

Response:

{"response":"WRITE_OK","type":144,"serial":"A1B2001C"}

Program a MOTOR_BWD block

{"cmd":"WRITE_BLOCK","type":145,"name":"Motor Bwd"}

Response:

{"response":"WRITE_OK","type":145,"serial":"A1B2001D"}

Program a MOTOR_LEFT block

{"cmd":"WRITE_BLOCK","type":146,"name":"Motor Left"}

Response:

{"response":"WRITE_OK","type":146,"serial":"A1B2001E"}

Program a MOTOR_RIGHT block

{"cmd":"WRITE_BLOCK","type":147,"name":"Motor Right"}

Response:

{"response":"WRITE_OK","type":147,"serial":"A1B2001F"}

Program a MOTOR_STOP block

{"cmd":"WRITE_BLOCK","type":148,"name":"Motor Stop"}

Response:

{"response":"WRITE_OK","type":148,"serial":"A1B20020"}

Using subtype and params:

Write a WAIT block with custom params (e.g. subtype=1 for variable wait, param1=10 for 10 units)

{"cmd":"WRITE_BLOCK","type":16,"subtype":1,"param1":10,"param2":0,"name":"Wait Custom"}

Response:

{"response":"WRITE_OK","type":16,"serial":"A1B20021"}

Minimal write – only required fields (defaults: channel=0, subtype=0, param1=0, param2=0, name="")

{"cmd":"WRITE_BLOCK","type":1}

Response:

{"response":"WRITE_OK","type":1,"serial":"A1B20022"}

Error cases:

Missing type field

{"cmd":"WRITE_BLOCK","channel":0,"name":"Oops"}

Response:

{"response":"ERROR","code":1,"message":"Missing or invalid 'type'"}

Invalid block type (0xFF is not a known type)

{"cmd":"WRITE_BLOCK","type":255,"name":"Bad"}

Response:

{"response":"ERROR","code":2,"message":"Write failed"}

## READ_BLOCK

Reads the 32-byte block header from the EEPROM on the specified channel and returns all fields as JSON.

| Field | Type | Required | Description |
| --- | --- | --- | --- |
| cmd | string | Yes | "READ_BLOCK" |
| channel | int | No | Mux channel 0–7 (default 0) |

### READ_BLOCK Examples

Read from default channel 0

{"cmd":"READ_BLOCK"}

Response:

{"response":"READ_DATA","type":1,"subtype":0,"param1":0,"param2":0,"serial":"A1B20000","name":"Start"}

Read from channel 3

{"cmd":"READ_BLOCK","channel":3}

Response:

{"response":"READ_DATA","type":80,"subtype":0,"param1":0,"param2":0,"serial":"A1B2000D","name":"If Button"}

Read from channel 5 (LED Red was written here)

{"cmd":"READ_BLOCK","channel":5}

Response:

{"response":"READ_DATA","type":114,"subtype":0,"param1":0,"param2":0,"serial":"A1B20015","name":"LED Red"}

Read from channel 7

{"cmd":"READ_BLOCK","channel":7}

Response:

{"response":"READ_DATA","type":116,"subtype":0,"param1":0,"param2":0,"serial":"A1B20017","name":"LED Blue"}

Read a blank/empty EEPROM (all 0xFF)

{"cmd":"READ_BLOCK","channel":4}

Response:

{"response":"READ_DATA","type":255,"subtype":255,"param1":255,"param2":255,"serial":"FFFFFFFF","name":""}

Read all 8 channels in sequence

{"cmd":"READ_BLOCK","channel":0}
{"cmd":"READ_BLOCK","channel":1}
{"cmd":"READ_BLOCK","channel":2}
{"cmd":"READ_BLOCK","channel":3}
{"cmd":"READ_BLOCK","channel":4}
{"cmd":"READ_BLOCK","channel":5}
{"cmd":"READ_BLOCK","channel":6}
{"cmd":"READ_BLOCK","channel":7}

## ERASE_BLOCK

Sets all 32 bytes of the block data region to 0xFF (blank). Verifies the erase by reading back.

| Field | Type | Required | Description |
| --- | --- | --- | --- |
| cmd | string | Yes | "ERASE_BLOCK" |
| channel | int | No | Mux channel 0–7 (default 0) |

### ERASE_BLOCK Examples

Erase channel 0

{"cmd":"ERASE_BLOCK","channel":0}

Response:

{"response":"ERASE_OK"}

Erase channel 5

{"cmd":"ERASE_BLOCK","channel":5}

Response:

{"response":"ERASE_OK"}

Erase default channel (0)

{"cmd":"ERASE_BLOCK"}

Response:

{"response":"ERASE_OK"}

Erase then verify it is blank

{"cmd":"ERASE_BLOCK","channel":2}
{"cmd":"READ_BLOCK","channel":2}

Response:

{"response":"ERASE_OK"}
{"response":"READ_DATA","type":255,"subtype":255,"param1":255,"param2":255,"serial":"FFFFFFFF","name":""}

Erase all 8 channels

{"cmd":"ERASE_BLOCK","channel":0}
{"cmd":"ERASE_BLOCK","channel":1}
{"cmd":"ERASE_BLOCK","channel":2}
{"cmd":"ERASE_BLOCK","channel":3}
{"cmd":"ERASE_BLOCK","channel":4}
{"cmd":"ERASE_BLOCK","channel":5}
{"cmd":"ERASE_BLOCK","channel":6}
{"cmd":"ERASE_BLOCK","channel":7}

## VERIFY_BLOCK

Reads the EEPROM on the specified channel and checks two things: (1) the XOR checksum matches, and (2) the block type is a recognized ID. Returns match: true if both pass, false otherwise.

| Field | Type | Required | Description |
| --- | --- | --- | --- |
| cmd | string | Yes | "VERIFY_BLOCK" |
| channel | int | No | Mux channel 0–7 (default 0) |

### VERIFY_BLOCK Examples

Verify a properly programmed block on channel 0

{"cmd":"VERIFY_BLOCK","channel":0}

Response:

{"response":"VERIFY_OK","match":true}

Verify channel 5 after writing LED_RED

{"cmd":"VERIFY_BLOCK","channel":5}

Response:

{"response":"VERIFY_OK","match":true}

Verify a blank EEPROM (will fail – type 0xFF is not valid)

{"cmd":"VERIFY_BLOCK","channel":4}

Response:

{"response":"VERIFY_OK","match":false}

Verify default channel

{"cmd":"VERIFY_BLOCK"}

Response:

{"response":"VERIFY_OK","match":true}

Write then immediately verify

{"cmd":"WRITE_BLOCK","channel":6,"type":128,"name":"Beep"}
{"cmd":"VERIFY_BLOCK","channel":6}

Response:

{"response":"WRITE_OK","type":128,"serial":"A1B20030"}
{"response":"VERIFY_OK","match":true}

## BATCH_PROGRAM

Programs multiple blocks in sequence from a single command. Each entry in the blocks array can target a different channel. Between entries, the firmware waits for user confirmation (any keypress) to allow swapping in the next EEPROM if using a single-slot setup. Each block gets its own WRITE_OK/ERROR response.

| Field | Type | Required | Description |
| --- | --- | --- | --- |
| cmd | string | Yes | "BATCH_PROGRAM" |
| blocks | array | Yes | Array of block objects (same fields as WRITE_BLOCK minus cmd) |

Each object in blocks[] accepts:

| Field | Type | Required | Description |
| --- | --- | --- | --- |
| channel | int | No | Mux channel 0–7 (default 0) |
| type | int | Yes | Block Type ID |
| subtype | int | No | Block subtype (default 0) |
| param1 | int | No | Parameter 1 (default 0) |
| param2 | int | No | Parameter 2 (default 0) |
| name | string | No | Label, max 15 chars |

### BATCH_PROGRAM Examples

Batch program 2 blocks on different channels

{"cmd":"BATCH_PROGRAM","blocks":[{"channel":0,"type":1,"name":"Start"},{"channel":1,"type":114,"name":"LED Red"}]}

Response:

{"response":"WRITE_OK","type":1,"serial":"A1B20040"}
{"response":"WRITE_OK","type":114,"serial":"A1B20041"}

Batch program a simple program: Start → Beep → Wait 1s → LED Red

{"cmd":"BATCH_PROGRAM","blocks":[{"channel":0,"type":1,"name":"Start"},{"channel":1,"type":128,"name":"Beep"},{"channel":2,"type":16,"name":"Wait 1s"},{"channel":3,"type":114,"name":"LED Red"}]}

Response:

{"response":"WRITE_OK","type":1,"serial":"A1B20042"}
{"response":"WRITE_OK","type":128,"serial":"A1B20043"}
{"response":"WRITE_OK","type":16,"serial":"A1B20044"}
{"response":"WRITE_OK","type":114,"serial":"A1B20045"}

Batch program a loop: Start → Loop Forever → LED Green → Wait 2s → LED Off → Wait 2s

{"cmd":"BATCH_PROGRAM","blocks":[{"channel":0,"type":1,"name":"Start"},{"channel":1,"type":64,"name":"Loop Forever"},{"channel":2,"type":115,"name":"LED Green"},{"channel":3,"type":17,"name":"Wait 2s"},{"channel":4,"type":113,"name":"LED Off"},{"channel":5,"type":17,"name":"Wait 2s"}]}

Batch program with conditional: Start → If Button → Play Happy → Else → Play Sad → End If

{"cmd":"BATCH_PROGRAM","blocks":[{"channel":0,"type":1,"name":"Start"},{"channel":1,"type":80,"name":"If Button"},{"channel":2,"type":130,"name":"Play Happy"},{"channel":3,"type":49,"name":"Else"},{"channel":4,"type":131,"name":"Play Sad"},{"channel":5,"type":50,"name":"End If"}]}

Batch program a motor sequence: Start → Motor Fwd → Wait 5s → Motor Left → Wait 2s → Motor Stop

{"cmd":"BATCH_PROGRAM","blocks":[{"channel":0,"type":1,"name":"Start"},{"channel":1,"type":144,"name":"Motor Fwd"},{"channel":2,"type":18,"name":"Wait 5s"},{"channel":3,"type":146,"name":"Motor Left"},{"channel":4,"type":17,"name":"Wait 2s"},{"channel":5,"type":148,"name":"Motor Stop"}]}

Batch program all on same channel (single-slot setup, swap between each)

{"cmd":"BATCH_PROGRAM","blocks":[{"type":1,"name":"Start"},{"type":114,"name":"LED Red"},{"type":16,"name":"Wait 1s"},{"type":115,"name":"LED Green"}]}

Batch with one invalid type (3rd block will fail, others succeed)

{"cmd":"BATCH_PROGRAM","blocks":[{"channel":0,"type":1,"name":"Start"},{"channel":1,"type":128,"name":"Beep"},{"channel":2,"type":255,"name":"Bad Block"}]}

Response:

{"response":"WRITE_OK","type":1,"serial":"A1B20050"}
{"response":"WRITE_OK","type":128,"serial":"A1B20051"}
{"response":"ERROR","index":2,"message":"Write failed"}

# Common Workflows

## Program and verify a single block

Step 1: Write the block

{"cmd":"WRITE_BLOCK","channel":0,"type":1,"name":"Start"}

Step 2: Verify it

{"cmd":"VERIFY_BLOCK","channel":0}

Step 3: Read it back to confirm

{"cmd":"READ_BLOCK","channel":0}

## Reprogram an existing block

Step 1: Erase the old data

{"cmd":"ERASE_BLOCK","channel":3}

Step 2: Write new data

{"cmd":"WRITE_BLOCK","channel":3,"type":145,"name":"Motor Bwd"}

Step 3: Verify

{"cmd":"VERIFY_BLOCK","channel":3}

## Scan all 8 channels to see what is programmed

{"cmd":"READ_BLOCK","channel":0}

{"cmd":"READ_BLOCK","channel":1}

{"cmd":"READ_BLOCK","channel":2}

{"cmd":"READ_BLOCK","channel":3}

{"cmd":"READ_BLOCK","channel":4}

{"cmd":"READ_BLOCK","channel":5}

{"cmd":"READ_BLOCK","channel":6}

{"cmd":"READ_BLOCK","channel":7}

## Factory reset – erase all 8 channels

{"cmd":"ERASE_BLOCK","channel":0}

{"cmd":"ERASE_BLOCK","channel":1}

{"cmd":"ERASE_BLOCK","channel":2}

{"cmd":"ERASE_BLOCK","channel":3}

{"cmd":"ERASE_BLOCK","channel":4}

{"cmd":"ERASE_BLOCK","channel":5}

{"cmd":"ERASE_BLOCK","channel":6}

{"cmd":"ERASE_BLOCK","channel":7}

## Program a complete 8-block program across all channels

Example: Start → Loop Forever → LED Red → Beep → Wait 1s → LED Blue → Beep Long → Wait 2s

{"cmd":"BATCH_PROGRAM","blocks":[{"channel":0,"type":1,"name":"Start"},{"channel":1,"type":64,"name":"Loop Forever"},{"channel":2,"type":114,"name":"LED Red"},{"channel":3,"type":128,"name":"Beep"},{"channel":4,"type":16,"name":"Wait 1s"},{"channel":5,"type":116,"name":"LED Blue"},{"channel":6,"type":129,"name":"Beep Long"},{"channel":7,"type":17,"name":"Wait 2s"}]}

# Block Type IDs

Use the Decimal value in the "type" JSON field.

| Category | Name | Hex | Decimal |
| --- | --- | --- | --- |
| Events | START | 0x01 | 1 |
| Events | WHEN_BUTTON | 0x02 | 2 |
| Events | WHEN_SENSOR | 0x03 | 3 |
| Control | WAIT_1S | 0x10 | 16 |
| Control | WAIT_2S | 0x11 | 17 |
| Control | WAIT_5S | 0x12 | 18 |
| Control | REPEAT_2X | 0x20 | 32 |
| Control | REPEAT_3X | 0x21 | 33 |
| Control | REPEAT_5X | 0x22 | 34 |
| Control | IF | 0x30 | 48 |
| Control | ELSE | 0x31 | 49 |
| Control | ENDIF | 0x32 | 50 |
| Control | LOOP_FOREVER | 0x40 | 64 |
| Sensing | IF_BUTTON | 0x50 | 80 |
| Sensing | IF_LIGHT_HIGH | 0x51 | 81 |
| Sensing | IF_LIGHT_LOW | 0x52 | 82 |
| Sensing | IF_SOUND | 0x53 | 83 |
| Sensing | IF_DISTANCE | 0x54 | 84 |
| Sensing | IF_TEMP_HIGH | 0x55 | 85 |
| LED | LED_ON | 0x70 | 112 |
| LED | LED_OFF | 0x71 | 113 |
| LED | LED_RED | 0x72 | 114 |
| LED | LED_GREEN | 0x73 | 115 |
| LED | LED_BLUE | 0x74 | 116 |
| Sound | BEEP | 0x80 | 128 |
| Sound | BEEP_LONG | 0x81 | 129 |
| Sound | PLAY_HAPPY | 0x82 | 130 |
| Sound | PLAY_SAD | 0x83 | 131 |
| Motion | MOTOR_FWD | 0x90 | 144 |
| Motion | MOTOR_BWD | 0x91 | 145 |
| Motion | MOTOR_LEFT | 0x92 | 146 |
| Motion | MOTOR_RIGHT | 0x93 | 147 |
| Motion | MOTOR_STOP | 0x94 | 148 |

# Error Responses

On failure, all commands return:

{"response":"ERROR","code":<int>,"message":"<string>"}

| Code | Meaning | Cause |
| --- | --- | --- |
| 0 | Parse error | Invalid JSON, missing cmd field, unknown command, or buffer overflow |
| 1 | Invalid type | Missing or invalid type field in WRITE_BLOCK |
| 2 | Write failed | EEPROM write or post-write verification failed |
| 3 | Read failed | EEPROM read I²C communication failed |
| 4 | Erase failed | EEPROM erase or post-erase verification failed |
| 5 | Verify read failed | Could not read EEPROM during VERIFY_BLOCK |
| 6 | Batch error | Missing or invalid blocks array in BATCH_PROGRAM |

## Error Examples

Invalid JSON

this is not json

Response:

{"response":"ERROR","code":0,"message":"Invalid JSON"}

Missing cmd field

{"type":1}

Response:

{"response":"ERROR","code":0,"message":"Missing 'cmd' field"}

Unknown command

{"cmd":"DO_SOMETHING"}

Response:

{"response":"ERROR","code":0,"message":"Unknown command"}

Write with missing type

{"cmd":"WRITE_BLOCK","name":"Oops"}

Response:

{"response":"ERROR","code":1,"message":"Missing or invalid 'type'"}

Write with unrecognized type ID

{"cmd":"WRITE_BLOCK","type":200}

Response:

{"response":"ERROR","code":2,"message":"Write failed"}

# Status LED Behavior

The onboard WS2812 RGB LED (GPIO48) indicates the current state:

| State | Color | Pattern |
| --- | --- | --- |
| Idle | Off | LED is off, waiting for commands |
| Programming | Blue | Blinking while writing or erasing EEPROM |
| Success | Green | 3 flashes, then returns to idle |
| Error | Red | 3 flashes, then returns to idle |